STYRENE PARK OFFSITE Document Title: Lubricant List/Grease List

| R0 | 24-08-2024 | IFA | F.sh | M.O | A.M |
| --- | --- | --- | --- | --- | --- |
| Rev. | Issued Date | DESCRIPTION | PREPARED | CHECKED | APPROVED |

REVISION RECORD SHEET

| Page Page | Revisions |  |  |  |  |  |  |  | Page | Revisions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R0 | R1 | R2 | R3 | R4 | R5 | R6 |  |  | R0 | R1 | R2 | R3 | R4 | R5 | R6 |
| 1 | X |  |  |  |  |  |  |  | 41 |  |  |  |  |  |  |  |
| 2 | X |  |  |  |  |  |  |  | 42 |  |  |  |  |  |  |  |
| 3 | X |  |  |  |  |  |  |  | 43 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  | 44 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  | 45 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  | 46 |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  | 47 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  | 48 |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  | 49 |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  | 50 |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  | 51 |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  | 52 |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |  | 53 |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  | 54 |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  | 55 |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  | 56 |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |  | 57 |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |  | 58 |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  | 59 |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |
| 21 |  |  |  |  |  |  |  |  | 61 |  |  |  |  |  |  |  |
| 22 |  |  |  |  |  |  |  |  | 62 |  |  |  |  |  |  |  |
| 23 |  |  |  |  |  |  |  |  | 63 |  |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  |  |  | 64 |  |  |  |  |  |  |  |
| 25 |  |  |  |  |  |  |  |  | 65 |  |  |  |  |  |  |  |
| 26 |  |  |  |  |  |  |  |  | 66 |  |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  |  |  | 67 |  |  |  |  |  |  |  |
| 28 |  |  |  |  |  |  |  |  | 68 |  |  |  |  |  |  |  |
| 29 |  |  |  |  |  |  |  |  | 69 |  |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  |  |  | 70 |  |  |  |  |  |  |  |
| 31 |  |  |  |  |  |  |  |  | 71 |  |  |  |  |  |  |  |
| 32 |  |  |  |  |  |  |  |  | 72 |  |  |  |  |  |  |  |
| 33 |  |  |  |  |  |  |  |  | 73 |  |  |  |  |  |  |  |
| 34 |  |  |  |  |  |  |  |  | 74 |  |  |  |  |  |  |  |
| 35 |  |  |  |  |  |  |  |  | 75 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |  | 76 |  |  |  |  |  |  |  |
| 37 |  |  |  |  |  |  |  |  | 77 |  |  |  |  |  |  |  |
| 38 |  |  |  |  |  |  |  |  | 78 |  |  |  |  |  |  |  |
| 39 |  |  |  |  |  |  |  |  | 79 |  |  |  |  |  |  |  |
| 40 |  |  |  |  |  |  |  |  | 80 |  |  |  |  |  |  |  |

| EQUIPMENT | GRADE (ISO OR ASTM) | MAKER'S BRAND | VISCOSITY | INITIAL CHARGE | REPLACEMENT |  | MAKE UP |  | REMARKS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | VOLUME | INTERVAL | VOLUME | INTERVAL |  |  |
| Compressor RU0001A-C-01 RU0001B-C-01 | OIL ISO VG100 | CPI 1507-68 | 63 (@40℃) | 360 liter (TBC) | 360L (1*) | N/A (*2) | N/A (*2) | N/A (*2) | - Shaft seal leakage - oil filter replacement |
| Mian Motor None Drive End Bearing RU0001A-M-01 RU0001B-M-01 | Grease | ESSO Unirex N3 | 100 (TBC) | 30g (Drive end) | 30g | Conintuous Operation, 3000 Hours (*4) | N/A (*2) | N/A (*2) | Self lubrication |
| Main Motor Non Drive End Bearing RU0001A-M-01 RU0001B-M-01 | Grease | ESSO Unirex N3 | 100 (TBC) | 30g (Non Drive end) | 30g | Conintuous Operation, 3000 Hours (*4) | N/A (*2) | N/A (*2) | Self lubrication |
| Oil Pump Motor RU0001A-M-04 RU0001B-M-04 | Grease | N/A | N/A | N/A | N/A | N/A | N/A | N/A | Lifetime lubricated bearings. No re-greasing required |
| Notes: 1* Oil analysis should be performed every 6 months. Oil should be replaced if oil analysis indicates that change is needed. 2* There is no fixed interval. It’s advisable to check on regulary basis the oil level on the level guage of the oil separator for compressor and for main motor on side glass. 3* Replacement of total volume of oil is advised to be replaced every 3-4 years due to wear and reduction of the oil quality itself. 4* Permanent lubication. |  |  |  |  |  |  |  |  |  |