MATERIAL REQUISITION FOR PIPING SPOOLS AND PACKAGE ASSEMBLY

Tabulation of Revised Pages

| Page | Revision |  |  |  |  |  |  |  | Page | Revision |  |  |  |  |  |  |  | Page | Revision |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 01 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 29 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 34 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table of Contents

1.	Scope	4

2.	Definitions	4

2.1	Abbreviations	4

3.	References	4

3.1	General	4

3.2	Code and standards	4

3.3	Project Documents	5

4.	Inquired Items	6

4.1	Item Name and Tag Number	6

5.	Scope of Work and Supply	6

5.1	Scope of Supply	6

5.2	Scope of Work	8

6.	Design	11

6.1	General	11

6.2	General Notes	12

6.3	Piping	12

6.4	Assembly	12

7.	Documentation	12

7.1	General	12

7.2	Conflict in purchaser documents	13

7.3	Language of documentation	13

7.4	Unit of measurement	13

7.5	Documents shall be submitted by vendor	13

8.	Inspection and Tests	16

8.1	General	16

8.2	Traceability Levels	17

8.3	NDE Qualification	17

9.	Guarantee	17

# Scope

This document covers, material supply, manufacturing, assembling, Inspection, testing, delivery and all other activities that is required for supply of Piping inside the package to successful startup of the package.

This Requisition sets out the minimum requirements and does not relieve the Vendor of his full responsibility for the reliable manufacturing of supplied items. Therefore, the Vendor will be liable for the correct operation of any auxiliary elements involved. In the preparation of his proposal, the Vendor shall fulfill all the provisions of this Requisition even if they differ from vendor standard supply. Whenever the Vendor cannot comply with the present Requisition for technical reasons, he shall state it in detail in the chapter of the deviations.

# Definitions

## Abbreviations

| Abbreviation | Meaning |
| --- | --- |
| NDT | None Destructive Test |
| QA | Quality assurance |
| WPS/PQR | Welding Procedure Specification/Procedure Qualification Record |
| CD | Compact Disc |
| QCP | Quality Control Plan |
| MOM | Minuets Of Meeting |
| PMI | Positive Metal Identification |

# References

## General

All documents, codes and standards referenced in this requisition, shall form an integral part of this Requisition and shall be followed where applicable. For project documents, latest revision of the documents up to date of kickoff meeting shall be followed.

For codes and standards, latest revision up to date of order shall be followed. Purchaser reserve the right to request the vendor to use and follow an older revision of a code or standard due to his requirements.

## Code and standards

| No. | Code/Standard Number | Code/Standard Title |
| --- | --- | --- |
| 1 | ASME B31.3 Edition 2022 | Process Piping |
| 2 | ASME Section V, Edition 2021 | Nondestructive Examination |
| 3 | ASME Section IX, Edition 2021 | Welding and Brazing Qualifications |
| 4 | ASTM/ASME Section II, Edition 2021 | Materials |
| 5 | BS EN 10204 (3.1) | Metallic Products - Types Of Inspection Documents |
| 6 | ASME B16.5 | Pipe Flanges And Flanged Fittings: Nps 1/2 Through Nps 24 Metric/Inch Standard |
| 7 | ASME B16.47 Series B | Large Diameter Steel Flanges Nps 26 Through Nps 60 Metric/Inch Standard |
| 8 | ASME B16.9 | Factory-Made Wrought Butt welding Fittings |
| 9 | ASME B16.20 | Metallic Gaskets For Pipe Flanges Ring-Joint, Spiral-Wound, And Jacketed |
| 10 | ASME B16.21 | Nonmetallic Flat Gaskets For Pipe Flanges |
| 11 | ASME B1.1 | Unified Inch Screw Threads |
| 12 | ASME B18.2.1 | Square, Hex, Heavy Hex, And Askew Head Bolts And Hex, Heavy Hex, Hex Flange, Lobed Head, And Lag Screws (Inch Series) |
| 13 | ASME B18.2.2 | Nuts For General Applications: Machine Screw Nuts, Hex, Square, Hex Flange, And Coupling Nuts (Inch Series) |
| 14 | ASME B36.19 | Stainless Steel Pipe |
| 15 | ASME B36.10 | Welded And Seamless Wrought Steel Pipe |
| 16 | AWS | American National Standard for Welding |

## Project Documents

| No. | Document Title | Document Number | Revision | Remarks |
| --- | --- | --- | --- | --- |
| A | General Documents : |  |  |  |
| 1 | Painting Procedure | #### EI027-HSE-VD – QC– PRO– 002 | 00 |  |
| 2 | Final Data Book Preparation Instruction |  | 00 |  |
|  | Packing, Shipping, Marking Procedure |  | 00 |  |
| 3 | Deviation List |  |  |  |
| 4 | Approved Vendor List |  | 08 | NPC, POGC, MOP |
| B | Technical Documents: |  |  |  |
| B-1 | Piping: |  |  |  |
| 1 | Isometrical drawings |  | 00 |  |
| 2 | Piping Orthos & Related Supports |  | 00 |  |
| 3 | 3D Model | EI027-HSE-VD – PI– MOD– 002 | 00 |  |
| 4 | Line List |  | 00 |  |
| 5 | Valve List |  | 00 |  |
| 6 | MTO for Pipe, Fitting, Flange |  | 00 |  |
| 7 | Inspection and Test Plan for Piping |  | 00 |  |
| 8 | Standart U Clamp |  | 00 |  |
| 9 | General Arrangement Drawing | EI027-HSE-VD – PI– DWG– 001 | 00 |  |
| 10 | Lifting Plan |  | 00 |  |
| B-2 | Process: |  |  |  |
| 1 | Piping and Instrument Diagram | EI027-HSE-VD-PR-PID-002 | 02 |  |
| B-3 | Strcture: |  |  |  |
| 1 | Chiller Package Skid | EI027-HSE-VD – ST– DWG– 001 | 02 |  |
| B-4 | Instrument: |  |  |  |
| 1 | Instrument List |  | 00 |  |
| 2 | Instrument Cable List |  | 01 |  |
| 3 | ISO for Tubing |  | 00 |  |
| 4 | MTO for Tubing |  | 00 |  |
| 5 | Hook Up |  | 00 |  |
| 6 | Cable Tray |  | 00 |  |
| B-5 | Assembly: |  |  |  |
| 1 | Inspection and Test Plan For Package Assembly |  | 00 |  |

# Inquired Items

## Item Name and Tag Number

| No. | Item Description | Item Tag Number | Number Required | Type |
| --- | --- | --- | --- | --- |
| 1 | Piping Spool |  | 2 Unit | Piping |
| 2 | Chiller Skid Assembly |  | 2 Unit | Assembly |

# Scope of Work and Supply

## Scope of Supply

Vendor proposal shall include, but not be limited to, equipment and materials specified in the following: Items marked with “vendor” in column of " Included in supply scope of:" hereunder shall be within vendor Scope of supply.

| No. | Item Description | Included In Supply Scope of: |
| --- | --- | --- |
| A | Piping |  |
| 1 | Supply of All Required Materials as Per Isometric Drawing, including but not limited to: Pipes, Flanges, Fitting, Olets, etc | Vendor |
| 2 | Supply of Supports For Piping as per Structure Drawing | Vendor |
| 3 | Supply of Cable Trays as per Drawing | Vendor |
| 4 | Supply of Supports For Cable Trays as per Structure Drawing | Vendor |
| 5 | Teflon (6 mm) Between Pipe and Pipe Support | Vendor |
| 6 | U bolt including 2 nuts | Vendor |
| 7 | Gasket, Bolts, Nuts and Washers as per Isometric | Vendor |
| 8 | Welding Rod for Shop Fabrication | Vendor |
| 9 | Welding Rod for site assemblies (if required) | Vendor |
| 10 | Cover the open ends with metal blank (Min. 6 mm) for Interconnection Pipes and other connection as battery limit. | Vendor |
| 11 | Gasket, Bolt and Nuts and washers for Interconnection Pipes for Transport | Vendor |
| 12 | Silica Gel for for Interconnection Pipes for Transport | Vendor |
| 13 | External Surface Painting | Vendor |
| 14 | Touch-up paint for repair work at site | Vendor |
| 15 | Valves | Others |
| 16 | Tag plate (Name Plate) for SS Manual Valves | Vendor |
| 17 | Instrument | Others |
| 18 | Insulation | Others |
| 19 | Fire Proofing | Others |
| 20 | 3.1 Certification for pressure part & 2.2 for Non-Pressure part materials | Vendor |
| 21 | All certificates requested in this MR | Vendor |
| 22 | Final Data Book | Vendor |
| B | Instrument |  |
| 1 | Supply of Cable Trays (SS316L) as per Drawing | Vendor |
| 2 | Supply of Supports For Cable Trays as per Structure Drawing | Vendor |
| 3 | Supply of Earth bar (2 earth bars per skid) | Vendor |
| 4 | Jumper for cable tray earthing | Vendor |
| 5 | Earth Boss (2 per skid) | Vendor |
| 6 | Instrument Consumable | Vendor |
| 7 | Locktite 577 | Vendor |
| 8 | Teflon | Vendor |
| 9 | Tag plate for Instrument Cable | Vendor |
| 10 | Bolt, Nuts and Washer for Installation of Junction Boxes | Vendor |
| 11 | Tubes | Others |
| 12 | Tube Fittings | Others |
| 13 | Tube Clamp | Others |
| 14 | Instrument Cable | Others |
| 15 | Earth Cable | Others |
| 16 | Cable Gland | Others |
| 17 | All Instruments | Others |
| 18 | Needls Valves | Others |
| 19 | Slonoid Valves | Others |
| 20 | Control Valvers | Others |
| 21 | Sight Glass | Others |
| 22 | Strainers | Others |
| 23 | Junction Boxes | Others |
| 24 | Heat Tracing | Others |

## Scope of Work

| No. | Item Description | Included in Work Scope of: |
| --- | --- | --- |
| A | Piping |  |
| 1 | Preparation of Piping Shop Drawings | Vendor |
| 2 | Material Procurement | Vendor |
| 3 | Fabrication (including all required welding) of Piping Spools at Shop | Vendor |
| 4 | Pipe Support Welding to Structure as per Structure Drawing | Vendor |
| 5 | Welder Performance Qualification Tests | Vendor |
| 6 | Inspection And Tests | Vendor |
| 7 | NDE | Vendor |
| 8 | Inside Cleaning | Vendor |
| 9 | Primer Painting | Vendor |
| 10 | Intermediate Painting | Vendor |
| 11 | Final Painting / Coating / Color approval | Vendor |
| 12 | Touch-up paint for repair work at site | Vendor |
| 13 | Chemical Cleaning, Pickling & Passivation For Stainless Steel Parts | Vendor |
| 14 | Positive Material Identification (PMI) For Non-Carbon Steel Material | Vendor |
| 15 | Hydrotest of Piping Spools | Vendor |
| 16 | Air Drying after Hydrotest | Vendor |
| 17 | Inland Shipping Marking / Packing | Vendor |
| 18 | Preparation For Shipment | Vendor |
| 19 | Ex-Work Delivery | Vendor |
| 20 | As Built Isometric Drawing | Vendor |
| 21 | Fianl Dta Book | Vendor |
| B | Instrument |  |
| 1 | Wiring of All Instruments as per Instrument list to Junction Box | Vendor |
| 2 | Wiring of Control Valve as per Instrument list to Junction Box | Vendor |
| 3 | Wiring of Solenoid Valves as per Instrument list to Junction Box | Vendor |
| 4 | Instrument End cable preparation | Vendor |
| 5 | Earthing of all Instruments, equipments, Valves, etc | Vendor |
| 6 | Earth cable End cable preparation | Vendor |
| 7 | Tubing of Oil recovery as per provided ISO and drawings | Vendor |
| 8 | Instrument Air Tubing as per provided ISO and drawings | Vendor |
| 9 | Junction Boxes Installation | Vendor |
| 10 | Termination inside Instruments | Vendor |
| 11 | Termination inside Junction Box's | Vendor |
| 12 | Function test of Instruments | Vendor |
| 13 | Cable Tray routing | Vendor |
| 14 | Cable Tray installation | Vendor |
| 15 | Connection tubing to Blind Flanges | Vendor |
| 16 | Cable Gland Installation | Vendor |
| 17 | Chemical Cleaning, Pickling & Passivation For Stainless Steel Parts | Vendor |
| C | Assembly |  |
| 1 | Mechanical Assembly including Assembly and installation of Piping as per project documents | Vendor |
| 2 | Assembly of all tubing and fitting as per provided ISO and drawings as per project documents | Vendor |
| 3 | Installation of U Bolt | Vendor |
| 4 | Touch up painting | Vendor |
| 5 | Detail study & fabrication | Vendor |
| 6 | NDE | Vendor |
| 7 | Inside cleaning | Vendor |
| 8 | Chemical Cleaning, Pickling & Passivation For Stainless Steel Parts | Vendor |
| 9 | Installation of bolts, nuts, washer and gaskets as per ISO and other project documents | Vendor |
| 10 | Insulation | Others |
| 11 | Fire Proofing | Others |
| 12 | Free Issued Material | Others |
| 13 | Costum Clearance on Free Issued Materail | Vendor |
| 14 | Delivery from Bandar Abbas to Vendor Shop (Client will deliver Free Issued Material in CFR Bandar Abbas Condition) | Vendor |
| 515 | Installation of Level Transmitter | Vendor |
| 16 | Installation of Level Gauge | Vendor |
| 17 | Installation of Control Valve | Vendor |
| 18 | Installation of Manual Valves | Vendor |
| 19 | Installation of Ejector | Vendor |
| 20 | All Wiring Check | Vendor |
| 21 | Earthing Check | Vendor |
| 22 | Wiring to Bar Check | Vendor |
| 23 | Documentation | Vendor |
| 24 | Inspection and test as per ITP | Vendor |
| 25 | Nitrogen Leak Test of the Package (with 22 barg), to be quoted separately | Vendor |
| 26 | N2 purging of the Package (with 1 barg) | Vendor |
| 27 | NDE report, Painting reports, Hydro test reports, leat test report, other reports | Vendor |
| 28 | As Built Drawing | Vendor |
| 29 | Packing List | Vendor |
| 30 | Package Packing, to be qouted separately for belw options: -Wooden Box - Tarpaulin Packing | Vendor |
| 31 | 3.1 Certification for pressure part & 2.2 for Non-Pressure part materials | Vendor |
| 32 | All certificates requested in this MR | Vendor |
| 33 | Inspection release note | Vendor |
| 34 | Load skid on Truck | Vendor |
| 35 | Final Data Book | Vendor |

# Design

## General

All of referenced and attached standards or project documents shall be used and followed strictly when applicable including all typical drawing or details.

## General Notes

### All Documents / Drawings Shall Be Written In English.

### All Vendor Drawings And Documents Should Be Provided In Purchaser Title Block.

### All E-Files Should Be Provided In Original Format (E.G AutoCAD, Word, Excel, Etc.)

## Piping

### 15 % RT (Spot) is considered for pipe spools (Butt Joints) only.

### All the joints shall be visually inspected and DP testing will be done.

### 3.1 Certification is considered for all piping materials including Gasket, Bolts, Nuts

### Flanges Face Finishing Shall Be Smooth With 125 Micro Inch Minimum To 250 Micro Inch Maximum, As Per ASME B16.5/B16.47.

### Flange Bolt Holes Shall Be Straddled North/South C.L. (Or Horizontal C.L.).

### All material shall be supplied from latest Revision of NPC, POGC and MOP approved vendor list

### Fabrication Tolerances for piping shall be as per ASME B31.3 and other documents in material requisition.

## Assembly

### Free issued material will be supplied by Clinet with CFR Bandar Abbas condition. Costum clearance and delivery to Vendor shop is in Vendor responsibility

### Package assembly shall be done as per provided ITP and other project specification and documents. Level

### All flanges shall be straddled to north south to be match with valves and instruments.

### Installation of pipe on relevant support shall be done properly to avoid gap between support and skids. Pre assembly between support and skid and pipe to be done before fianl assembly.

### Required volume of Nitrogen for Leak test of package shall be take into consideration by vendor during proposal stage.

### Free issued materials are out of vendor scope of supply and will be provided by Client.

### In case of discrepancy between provided MTO for piping and Isometric, please inform client.

# Documentation

## General

During Equipment / Material Service Purchase Order implementation stage:

Issue of the Job QA plan

Issue of working procedures for purchaser review within one month after order.

Implementation of the working procedure during order execution

Performance of vendor Management Audit Program

Vendor shall provide the Copy of vendor quality manual / quality plan. Quality Plan shall be subject to presentation, discuss and clarification with purchaser, and Company as necessary, prior to incorporation into contract.

## Conflict in purchaser documents

In case of conflict between purchaser documents, Vendor shall inform the Purchaser about the conflict. For important conflicts, vendor shall wait for purchaser clarification before proceeding. For other events, documents priority order as specified hereinafter should be used. When conflict is not clarified between purchaser and vendor before order, the purchaser has the right to select the most stringent requirements without accepting any cost or time impact.

## Language of documentation

All documents shall be written in the English. Persian may be used for local letters and minutes of meeting if approved by purchaser. All documents in other languages shall be translated to English before submittal.

## Unit of measurement

SI units shall be used as far as possible except for followings:

- Pipe and flange nominal diameter should be represented in inch.

## Documents shall be submitted by vendor

| LIST OF DOCUMENTS REQUIRED FROM VENDOR |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NO. | DESCRIPTION | AT THE TIME OF BID Q/TYPE | AFTER ORDERING |  |  |  |  |
|  |  |  | DRAWINGS AND DOCUMENTS (NOTE 10) |  |  |  | AT THE FINAL INSP. (Note 4) |
|  |  |  | Q/TYPE (Note 1) | DEL. TIME (Note 2) | PURPOSE (Note 3) | FINAL ISSUE (Note 1) |  |
| A | General: |  |  |  |  |  |  |
| 1 | Manufacturing Schedule Per Equipment (Including Design, Procurement, Fabrication, Test And Packing, Delivery) | 4/C | 4/C+1/CD | Monthly | R | - | - |
| 2 | Sub-Vendor/Sub-Supplier List (Note 6) | 4/C | - | - | R | - | - |
| 3 | Country of Origins of Material | 4/C | 4/C+1/CD | With Material Certificate | I | - | - |
| 4 | Vendor’s Progress Report | - | 4/C | Monthly | R | - | - |
| 5 | Material Certificates / Certificate of Origin | - | 4/C+1/CD | 3 W Before Fabrication Start | I | 10/C+2/CD | X |
| 6 | Copy Of Un-priced Sub-Vendor/Sub-Supplier Orders | - | 4/C+1/CD | 1 W After Sub-Order Placement | - | - | - |
| 7 | Vendor Quality Control Plan | - | 4/C+1/CD | 4 W | R | 10/C+2/CD | X |
| 8 | Inspection and Test Plan | - | 4/C+1/CD | 4 W | R | 10/C+2/CD | X |
| 9 | Welding Procedure Specifications And Procedure Qualification Records (WPS & PQR) Including General Arrangement Drawings And Related Weld Or Welding Map For All Welding | - | 4/C+1/CD | 6 W Before Welding | - | 10/C+2/CD | X |
| 10 | Welders Qualification Certificates | - | 4/C+1/CD | 3 W Before Welding | - | 10/C+2/CD | X |
| 11 | Testing / NDT Procedures | - | 4/C+1/CD | 6 W Before Test | R | 10/C+2/CD | X |
| 12 | Painting Procedure | - | 4/C+1/CD | 6 W Before Painting | R | 10/C+2/CD | X |
| 13 | Pickling And Passivation Procedure | - | 4/C+1/CD | 6 W Before Pickling | R | 10/C+2/CD | X |
| 14 | NDT Personal Qualification Certificate | - | 4/C+1/CD | 6 W Before Test | R | 10/C+2/CD | X |
| 15 | NDT Map | - | 4/C+1/CD | 6 W Before Test | R | 10/C+2/CD | X |
| 16 | Testing / NDT Records | - | 4/C+1/CD | 2 W After Test | R | 10/C+2/CD | X |
| 17 | PMI Procedure | - | 4/C+1/CD | 6 W Before Test | R | 10/C+2/CD | X |
| 18 | PMI Record | - | 4/C+1/CD | 2 W After Test | R | 10/C+2/CD | X |
| 19 | Hydrotest Procedure | - | 4/C+1/CD | 6 W Before Test | R | 10/C+2/CD | X |
| 20 | Package Leak Test Procedure |  | 4/C+1/CD | 6 W Before Test | R | 10/C+2/CD | X |
| 21 | N2 purging procedure |  | 4/C+1/CD | 6 W Before Test | R | 10/C+2/CD | X |
| 22 | Hydrotest Certificates | - | 4/C+1/CD | 2 W After Treat | R | 10/C+2/CD | X |
| 23 | Other Shop Test Reports | - | 4/C+1/CD | 2 W Before Test | R | 10/C+2/CD | X |
| 24 | Packing Lists and Packing Sketch | - | 4/C+1/CD | Upon Shipment | R | 10/C+2/CD | X |
| 25 | Material + Mill Certificates And Acceptance Certificates | - | 10/C+2/ CD | 1 W Before Delivery | R | 10/C+2/CD | X |
| 26 | Package Inspection Release Certificates | - | 10/C+2/ CD | 2 W After Delivery | R | 10/C+2/CD | X |
| 27 | As-Built Drawings | - | 10/C+2/ CD | 2 W Before Delivery | R | 10/C+2/CD | X |
| 28 | List of Documents to Be Included in Vendor Data Book | - | 4/C+1/CD | 2 M Before Delivery | R | - | X |
| 29 | Vendor Data Book | - | - | 2 W After Delivery | R | 10/C+4/CD | X |
| Notes: |  |  |  |  |  |  |  |
| (1) | Q=QUANTITY, C=COPY, T=TRANSPARENT, P=POLYESTER, CD=COMPACT DISC, E=E-FILE |  |  |  |  |  |  |
| (2) | W=WEEK AFTER ORDER PLACEMENT, M=MONTH AFTER ORDER PLACEMENT |  |  |  |  |  |  |
| (3) | R=FOR REVIEW, I=FOR INFORMATION |  |  |  |  |  |  |
| (4) | X=DOCUMENTS TO BE SUBMITTED AT THE FINAL INSPECTION / DATA BOOK |  |  |  |  |  |  |
| (5) | PRICED COPY + UNPRICED COPY |  |  |  |  |  |  |
| (6) | PURCHASER HAS THE RIGHT TO REJECT SUB-VENDORS/SUB-SUPPLIERS IF HIS CAPABILITY IS JUDGED POOR OR NOT PROVEN. |  |  |  |  |  |  |
| (7) | NO DRAWING / DOCUMENT COULD BE REVIEWED BEFORE RECEIVING VPIS. |  |  |  |  |  |  |
| (8) | ALL DRAWINGS SHOULD INCLUDE WEIGHTED PART LIST. |  |  |  |  |  |  |
| (9) | ORIGINAL E_FILE SHOULD BE PROVIDED FOR FEA |  |  |  |  |  |  |
| (10) | ORIGINAL E_FILE SHOULD BE PROVIDED. |  |  |  |  |  |  |

# Inspection and Tests

## General

Inspection and tests shall be made by Vendor in accordance with requirements specified in this Requisition and/or any Engineering Specification, code or standard referenced in this requisition.

Shop witness test shall be carried out in accordance with the documents approved by Purchaser. Any witness/testing/inspection without the approved document shall be invalid.

No protection, paint or other filling shall be applied to the contract works before they have been inspected, tested and approved.

Manufacturing Data Record for Equipment shall be prepared in accordance with applicable specification and shall be completed at the final stage of inspection/test for Purchaser's inspector's review prior to shipment of items.

## Traceability Levels

The following levels shall apply to the traceability of certification:

Level 1 – EN 10204-3.2 Critical items

Level 2 – EN 10204-3.1 Normal level of assurance

Level 3 – EN 10204-2.2 Non- critical items

## NDE Qualification

All personnel performing NDE functions must be adequately qualified. The NDE operators qualifications shall form part of all Vendor quality plans and therefore be approved by the purchaser before manufacture works by the inspector. The foregoing shall also apply to all sub- manufacturers.

# Guarantee

Vendor shall guarantee the operation of the equipment to the specified design conditions. Vendor shall warrant the materials and workmanship of the piping spool for a period of 12 months after successful start-up or eighteen (18) months from the date of certificate of mechanical completion. below items shall be considered by vendor. Under this warranty, Vendor shall replace all materials, parts, and assemblies found to be defective during the guarantee period. Vendor shall also, if required, supply the services of an experienced engineer to supervise the necessary repairs and replacements.